BARDIC – TECHNICAL SPECIFICATIONS

VENUE DETAILS:

Proscenium Height		4.2m

Proscenium Width		10m

Stage Depth			8.3m

Stage Width			10.5m (variable, depending on masking)

Height of Stage		1m

Max. Performance Area	87m sq.

Grid Height			4.3m

Rake				No

Stage Floor Material		Plywood

Fixing to Stage Floor		Yes

Stage Left Wing Area		7.5m@DS, 4m@US

Stage Right Wing Area	5m@DS, 4m@CS, 2m@US
Scene Dock Storage Area	SL wings, if clear

Stage Crossover		Substage

Blackout Facility		Good

Dock Door Openings		2.1m high x 1.6m wide
Truck Access to Dock		Directly outside stage door

Travel Distance from Dock	5m, up slight incline

Loading Platform Height	Nil

Orchestra Pit			No

RIGGING /OVERHEAD SUSPENSION:

Flytower			No

Grid Height			4.3m comprises steel roof truss sections on which LX bars rest, and to which other bars can be attached. Some of the LX bars are potentially moveable.
Raise & Lower System	None

DIMMER CIRCUITS:

Dimmer Capacity	Qty	Location	Make/Model		Date Install

10A			48	Above DSR	Zero 88 Betapack	1990

10A			48	Above DSR	Zero 88 Betapack2	1999

LIGHTING BARS:

Quantity	Length	Circuit	Connector	Location

1		12m		12		15A		FOH 1.5m

1		12m		12		15A		1.5m US

1		12m		12		15A		2.5m US

1		12m		12		15A		4.5m US

1		12m		None		N/A		5.3m US

1		12m		12		15A		6m US

1		12m		None		N/A		6.8m US

1		12m		None		N/A		7.3m US

2		6m		6		15A		Up & Down at wings

2		4m		6		15A		Pros. booms

NOTE – Some of these bars are moveable, so may not be in specified position.

CONTROL:

Mains Patch			15A 96 ways of patching

Visiting Power Supply		C-form 63Amp 3-phase at DSR

Control Desk			ETC Element (Also ETC Express 125 & Strand 200)

Riggers Remote		No

Control Protocol		DMX512

LANTERN STOCK:

Quantity	Lantern

12		Selecon Pacific 28/50 Profiles 600w

3		ETC Source 4 50 degree Profiles 575w

4		ETC Source 4 Junior 25/50 degree Profiles 575w

10		CCT Minuette Profile 500w – Not all have shutters

12		Robert Juliat 310H 1kw Fresnels with Barndoors

16		Selecon Acclaim Fresnel 500w with Barndoors

16		CCT Minuette Fresnel 500w – No Barndoors

18		ETC Source 4 Par 575w

18		Selecon Lui Flood 1k

2		Followspots

16		7-Channel LED Pars

2		4-Way Bars of 7-Channel LED Pars

6		Mac 500 Spot

CABLES:

Quantity		Length	Core CSA	Connector

10 (approx)		5m		1.5sqmm	15A

10 (approx)		10m		1.5sqmm	15A

CONTROL ROOM:

Located in room at rear of auditorium.

ACCESS EQUIPMENT:

Min Height	Max Height	Type		Material

4m		7m		Zarges		Aluminium

2m		4m		Zarges		Aluminium

2m		2m		Stepladder	Aluminium

1m		1m		Stepladder	Aluminium